Design and Implementation of a Secure Dual-Stack Office Network Using GNS3

Network Design

[School]

[Course title]

Design and Implementation of a Secure Dual-Stack Office Network Using GNS3

Project #8

Project designed and developed by SUBASH SUBEDI.

Apex Digital Technology Services is a growing information technology and cybersecurity company that provides software development, network management, cloud services, penetration testing, DevOps, technical support and security-monitoring services to its customers. The company operates from a three-floor office building and currently employs approximately 180 permanent staff members, interns and technical contractors.

The ground floor contains Reception and Guest Services, the CCTV Security Room, meeting rooms, IP phones, printers and other Internet of Things devices. The first floor contains the Management and Network Administration team, general office staff, customer-support staff and internal wireless users. The second floor contains the Development and DevOps teams, Penetration Testing Laboratory, Network Operations and Monitoring Centre, and a secure server room containing virtualization servers, monitoring servers, backup systems, storage systems and public-facing services.

The company’s existing network was originally developed as a flat network in which users, servers, cameras, printers and wireless devices shared the same broadcast domain. As the company expanded, this design created performance, security and management problems. Broadcast traffic increased, sensitive systems were accessible from ordinary user devices, guest users could potentially discover internal resources and the company had no reliable method of controlling which Internet connection different departments used.

Senior management has therefore decided to redesign the complete office network using a secure, scalable and segmented architecture. The proposed network must support Local Area Network connectivity, dual Internet connections, IPv4 and IPv6, department-based VLANs, policy-based routing, automatic WAN failover, secure remote-access VPN, internal servers, monitoring systems, wireless infrastructure and firewall-based traffic isolation.

The simulated architecture is based on the functional design of the current office network. A MikroTik router performs inter-VLAN routing, IPv4 and IPv6 gateway functions, DHCP, DNS, firewalling, NAT, policy-based routing, WAN failover, VPN termination and network monitoring. A Layer 2 core switch transports the office VLANs through a two-link IEEE 802.3ad LACP trunk.

The company has subscribed to two Internet service providers. WAN1 is the primary connection for management systems, production servers, monitoring systems and the DMZ. WAN2 is the primary connection for staff, CCTV, guest wireless, wireless infrastructure and DevOps systems. Other departments use the main routing table, with WAN1 as the primary connection and WAN2 as the backup. This arrangement must be implemented for both IPv4 and IPv6.

The office network will use a VLAN-aware design. Management, CCTV, staff, penetration testing, guest wireless, servers, monitoring, backup, VoIP, DMZ, IoT, wireless infrastructure, storage and DevOps systems must each operate in separate VLANs and IP networks. The real office design uses the same functional separation to reduce broadcasts and control communication between services.

The project will be implemented in GNS3 using MikroTik CHR routers, an IOSvL2 switch, virtual PCs and Linux server appliances. The network will use a completely different laboratory IP-addressing plan and different VLAN numbers so that the simulation cannot conflict with the production office network.

You have been hired as the network and security engineer responsible for designing, implementing, securing, testing and documenting the proposed network. You must ensure that the completed network protects the confidentiality, integrity and availability of company information while remaining scalable, manageable and fault tolerant.

The firewall must follow a zone-based, least-privilege and default-deny approach. Established and related connections will be permitted, invalid traffic will be dropped, management access will be restricted, guest users will receive Internet-only access and only explicitly approved communication will be permitted between VLANs. This reflects the office firewall model, which combines RAW filtering, stateful input and forward rules, anti-spoofing controls and explicit inter-zone permissions.

Project Requirements

- Use GNS3 to design and implement the complete network solution.

- Use MikroTik CHR RouterOS 7.x as the office core router and Internet-edge security gateway.

- Use an IOSvL2 switch as the office Layer 2 core switch.

- Use two MikroTik CHR routers to simulate ISP1 and ISP2.

- Use an additional MikroTik CHR router to simulate the public Internet and recursive route-checking destinations.

- Use virtual PCs or lightweight Linux containers to represent clients in every VLAN.

- Use Ubuntu or Alpine Linux virtual machines to represent DNS, web, monitoring, DevOps and other internal servers.

- Design the network using a hierarchical model consisting of the Internet edge, core switching, access switching, wireless, server and monitoring layers.

- Connect the office router to the core switch using two physical interfaces configured as an IEEE 802.3ad LACP bond.

- Configure the LACP bond as a tagged trunk carrying all active office VLANs.

- Configure a separate untagged management access port so that the router can still be accessed when the core-switch trunk fails.

- Configure a blackhole or unused native-security VLAN on the trunk.

- Use a different VLAN ID for every network function.

- Use the IPv4 private address range 172.20.0.0/16 for the internal office network.

- Use the IPv6 ULA prefix fd42:7a61:9c20::/48 for the internal office network.

- Assign one IPv6 /64 network to every active VLAN.

- Configure the router address as the default IPv4 and IPv6 gateway for every VLAN.

- Configure IPv4 DHCP for all user and client VLANs.

- Configure IPv6 Router Advertisement and SLAAC for all active VLANs.

- Configure DHCP pools so that reserved infrastructure addresses do not overlap with dynamic addresses.

- Assign static IPv4 and IPv6 addresses to routers, switches, servers, access points, monitoring systems, storage systems and other infrastructure devices.

- Configure the Management VPN pool as 172.20.0.50–172.20.0.100.

- Configure the Management VLAN DHCP pool to begin at 172.20.0.101 so that it does not overlap with the VPN pool.

- Configure internal DNS services on the MikroTik router.

- Configure static DNS records for the router, switch, servers, monitoring platforms, reverse proxy and DevOps services.

- Advertise the VLAN gateway as the DNS server through both DHCPv4 and IPv6 Router Advertisement.

- Redirect Guest and IoT DNS requests to the router.

- Prevent Guest and IoT devices from bypassing the office DNS policy by using direct external DNS or DNS over TLS.

- Configure two routing tables named to-wan1 and to-wan2.

- Configure separate routing tables for router-generated WAN replies where required.

- Configure internal-destination routing exceptions before applying source-based policy-routing rules.

- Keep all traffic destined for internal IPv4 and IPv6 networks in the main routing table.

- Configure Management, Server, Monitoring and DMZ networks to use WAN1 as their primary Internet connection and WAN2 as backup.

- Configure CCTV, Staff, Guest, Wireless Infrastructure and DevOps networks to use WAN2 as their primary Internet connection and WAN1 as backup.

- Configure Pentest, Backup, VoIP, IoT and Storage networks to use the main routing table, with WAN1 primary and WAN2 backup.

- Implement the same routing policy for IPv4 and IPv6.

- Configure recursive route health checking instead of checking only the directly connected ISP gateways.

- Simulate public IPv4 and IPv6 probe addresses on the Internet router.

- Configure primary default routes with distance 1.

- Configure backup default routes with distance 2.

- Verify that each policy-routing table automatically changes to its backup ISP when the primary ISP becomes unavailable.

- Configure IPv4 source NAT or masquerade for all internal networks.

- Configure IPv6 NAT66 for the internal ULA prefix.

- Configure firewall interface lists and address lists representing WAN, LAN, Management, CCTV, Staff, Guest, Server, Monitoring, Backup, VoIP, DMZ, IoT, Wireless, Storage, DevOps and Pentest security zones.

- Configure RAW firewall rules to drop spoofed, malformed and known-invalid traffic before connection tracking.

- Drop traffic from private, loopback, multicast, documentation and other inappropriate source ranges arriving through WAN interfaces.

- Detect and restrict port scanners, brute-force sources, SYN floods and excessive connection attempts.

- Permit established, related and untracked traffic where appropriate.

- Drop invalid connections.

- Restrict router administration to the Management VLAN and approved VPN clients.

- Disable Telnet, FTP, unsecured API access, MAC Telnet, MAC WinBox, MAC Ping and unnecessary neighbor discovery.

- Configure SSH on a non-default TCP port.

- Permit DNS requests to the router only from authorized internal networks and VPN clients.

- Permit SNMP only from authorized monitoring servers.

- Configure Guest VLAN users for Internet access only.

- Block Guest users from accessing all internal IPv4 and IPv6 networks.

- Allow the Management VLAN to administer approved internal infrastructure.

- Allow Staff users to access only approved servers, directory services, DNS, web applications and the Internet.

- Isolate CCTV cameras from user and server networks.

- Permit only the monitoring servers to poll managed devices using SNMP, ICMP and approved monitoring protocols.

- Permit the DMZ reverse proxy to communicate only with approved backend services and ports.

- Restrict IoT and printer devices to DNS, NTP, required update services and approved print-server communication.

- Restrict the Pentest VLAN so that testing traffic cannot reach production networks unless an explicit temporary authorization rule is enabled.

- Permit Backup systems to communicate only with approved servers, storage systems and recovery services.

- Permit VoIP devices to communicate only with the PBX, DNS, NTP and approved voice services.

- Permit Storage systems to communicate only with authorized servers, backup systems and management hosts.

- Apply a final default-deny rule to inter-VLAN traffic.

- Apply a final default-deny rule to unsolicited WAN traffic.

- Configure L2TP over IPsec remote-access VPN on WAN1.

- Require IPsec protection for L2TP sessions.

- Use MSCHAPv2 authentication.

- Dynamically place connected VPN users in a firewall address list.

- Permit VPN clients to access only approved management and internal resources.

- Configure the simulated VPN pool inside the Management IPv4 subnet without overlapping the Management DHCP pool.

- Configure SNMP monitoring for the router and network devices.

- Configure logging for firewall, DHCP, DNS, routing, interface, VPN and system events.

- Configure Netwatch or equivalent monitoring tests for:

  - IPv4 through WAN1

  - IPv4 through WAN2

  - IPv6 through WAN1

  - IPv6 through WAN2

- Configure Network Time Protocol so that all routers and servers use consistent time.

- Configure system identity, warning banners, interface descriptions and comments on all routers and switches.

- Configure port security on access-switch interfaces where supported.

- Configure edge-port or PortFast behaviour only on ports connected to end devices.

- Configure BPDU Guard on appropriate access ports.

- Disable unused switch and router interfaces.

- Test IPv4 and IPv6 communication within each VLAN.

- Test authorized and unauthorized inter-VLAN communication.

- Test DHCPv4 address allocation.

- Test IPv6 Router Advertisement and SLAAC.

- Test DNS resolution and DNS redirection.

- Test policy-based routing from every VLAN.

- Test WAN1 and WAN2 failover.

- Test IPv4 NAT and IPv6 NAT66.

- Test firewall input-chain protection.

- Test firewall forward-chain segmentation.

- Test VPN establishment and VPN access permissions.

- Test SNMP polling, logging and Netwatch events.

- Verify that all configured services work according to the project requirements.

GNS3 VLAN, IPv4 and IPv6 Addressing Plan

| VLAN ID | VLAN name | Main purpose | IPv4 network and gateway | DHCP pool | IPv6 prefix and gateway | Primary route |
| --- | --- | --- | --- | --- | --- | --- |
| 200 | MGMT | Router, switch, administrator systems and VPN users | 172.20.0.0/24 - GW 172.20.0.1 | 172.20.0.101–172.20.0.254 | fd42:7a61:9c20:200::/64 - GW fd42:7a61:9c20:200::1 | WAN1, WAN2 backup |
| 210 | CCTV | Cameras, NVR and physical-security systems | 172.20.10.0/24 - GW 172.20.10.1 | 172.20.10.100–172.20.10.254 | fd42:7a61:9c20:210::/64 - GW fd42:7a61:9c20:210::1 | WAN2, WAN1 backup |
| 220 | STAFF | Employees, support staff and internal users | 172.20.20.0/23 - GW 172.20.20.1 | 172.20.20.101–172.20.21.254 | fd42:7a61:9c20:220::/64 - GW fd42:7a61:9c20:220::1 | WAN2, WAN1 backup |
| 230 | PENTEST | Penetration-testing and cybersecurity laboratory | 172.20.30.0/24 - GW 172.20.30.1 | 172.20.30.100–172.20.30.254 | fd42:7a61:9c20:230::/64 - GW fd42:7a61:9c20:230::1 | Main: WAN1, WAN2 backup |
| 240 | GUEST | Visitor and guest wireless access | 172.20.40.0/24 - GW 172.20.40.1 | 172.20.40.100–172.20.40.254 | fd42:7a61:9c20:240::/64 - GW fd42:7a61:9c20:240::1 | WAN2, WAN1 backup |
| 250 | SERVER | Virtualization hosts, directory and production servers | 172.20.50.0/24 - GW 172.20.50.1 | 172.20.50.100–172.20.50.254 | fd42:7a61:9c20:250::/64 - GW fd42:7a61:9c20:250::1 | WAN1, WAN2 backup |
| 260 | MONITORING | LibreNMS, Zabbix, Cacti, SIEM and syslog | 172.20.60.0/24 - GW 172.20.60.1 | 172.20.60.100–172.20.60.254 | fd42:7a61:9c20:260::/64 - GW fd42:7a61:9c20:260::1 | WAN1, WAN2 backup |
| 270 | BACKUP | Backup servers and disaster-recovery systems | 172.20.70.0/24 - GW 172.20.70.1 | 172.20.70.100–172.20.70.254 | fd42:7a61:9c20:270::/64 - GW fd42:7a61:9c20:270::1 | Main: WAN1, WAN2 backup |
| 280 | VOIP | IP phones, PBX and voice services | 172.20.80.0/24 - GW 172.20.80.1 | 172.20.80.100–172.20.80.254 | fd42:7a61:9c20:280::/64 - GW fd42:7a61:9c20:280::1 | Main: WAN1, WAN2 backup |
| 290 | DMZ | Reverse proxy and public-facing entry services | 172.20.90.0/24 - GW 172.20.90.1 | 172.20.90.100–172.20.90.254 | fd42:7a61:9c20:290::/64 - GW fd42:7a61:9c20:290::1 | WAN1, WAN2 backup |
| 300 | IOT | Printers, sensors and other IoT devices | 172.20.100.0/24 - GW 172.20.100.1 | 172.20.100.100–172.20.100.254 | fd42:7a61:9c20:300::/64 - GW fd42:7a61:9c20:300::1 | Main: WAN1, WAN2 backup |
| 310 | AP-INFRA | Wireless access points and wireless controllers | 172.20.110.0/24 - GW 172.20.110.1 | 172.20.110.100–172.20.110.254 | fd42:7a61:9c20:310::/64 - GW fd42:7a61:9c20:310::1 | WAN2, WAN1 backup |
| 320 | STORAGE | NAS, SAN and shared-storage systems | 172.20.120.0/24 - GW 172.20.120.1 | 172.20.120.100–172.20.120.254 | fd42:7a61:9c20:320::/64 - GW fd42:7a61:9c20:320::1 | Main: WAN1, WAN2 backup |
| 330 | DEVOPS | Git, CI/CD, automation and development platforms | 172.20.130.0/24 - GW 172.20.130.1 | 172.20.130.100–172.20.130.254 | fd42:7a61:9c20:330::/64 - GW fd42:7a61:9c20:330::1 | WAN2, WAN1 backup |
| 3999 | BLACKHOLE | Unused/native-security VLAN | No Layer 3 network | None | No Layer 3 prefix | None |

Special Address Allocation

| Purpose | Address or range |
| --- | --- |
| Router loopback IPv4 | 172.31.255.1/32 |
| Router loopback IPv6 | fd42:7a61:9c20:ffff::1/128 |
| VPN client range | 172.20.0.50–172.20.0.100 |
| WAN1 IPv4 router address | 198.51.100.2/30 |
| WAN1 IPv4 ISP gateway | 198.51.100.1 |
| WAN1 IPv6 router address | 2001:db8:100:1::2/64 |
| WAN1 IPv6 ISP gateway | 2001:db8:100:1::1 |
| WAN2 IPv4 router address | DHCP lease from 100.64.20.0/24 |
| WAN2 IPv4 ISP gateway | 100.64.20.1 |
| WAN2 IPv6 router address | DHCPv6 or 2001:db8:200:1::2/64 |
| WAN2 IPv6 ISP gateway | 2001:db8:200:1::1 |
| Internal IPv4 aggregate | 172.20.0.0/16 |
| Internal IPv6 aggregate | fd42:7a61:9c20::/48 |

The VLAN structure retains the same functional segmentation as the office design while using different VLAN IDs and non-production IPv4 and IPv6 ranges. The original office design similarly assigns dedicated subnets, gateways, DHCP pools and IPv6 /64 prefixes to each service VLAN.

Table of Contents

List of Figures

No table of figures entries found.

List of Tables

# Question Number 1 (Arial, 14, FontColor: Black)

## Sub heading (Arial, 13, FontColor: Black)

**Your content here (Arial, 12, Black) **

**All text must be justified**

**Line spacing : 1.5**

# Bibliography