Upgraded MPLS & VPLS Campus Network Implementation

MPLS & VPLS

SUBASH SUBEDI

[Course title]

Scenario

Islington College operates a multi-block campus network consisting of 9 academic and administrative blocks. The college owns a public IPv4 address range (160.30.132.0/24) and a public IPv6 address range (2001:df4:2b40::/48). The Private IPv4 range is used for router loopback interfaces within the MPLS backbone. The Private IPv6 range is used for router loopbacks, point-to-point core links, and VLAN services in the upgraded dual-stack design.

We are required to design, implement, and verify an MPLS-based core network using MikroTik routers in GNS3. The network must provide VPLS-based Layer 2 connectivity across all blocks while using a centralized DHCP server for end-device addressing. IPv6 will be deployed in dual-stack mode to support modern routing and services.

The LONDON_BLOCK is the main block, connected to the Internet (ISP). All other blocks connect to it directly and also through additional mesh links for redundancy and load balancing.

Network Architecture and IP Addressing Plan

Core Network Architecture

- All block routers act as MPLS core routers

- IS-IS is the IGP across all routers for IPv4 and IPv6

- MPLS with LDP must be enabled on all core and mesh links

- ECMP / load balancing must be implemented

- Each router must maintain 3 - 4 routed link connections with other core routers to ensure redundancy, optimal traffic distribution, and backbone load balancing

Routing & Connectivity

- iBGP used within the network for internal route distribution across core routers

- eBGP used for ISP connectivity simulation and external route exchange

- IPv6 dual-stack addressing and routing

- Public IPv6 reserved for ISP edge simulation and selected external services

- Private IP addressing used for backbone, loopbacks, point-to-point links, and internal VLAN services

Layer 2 Services (VPLS)

- Multiple VPLS domains: Student, Staff, CCTV, Voice, Management, Server, and Guest Wi-Fi

- VPLS must be used to extend VLANs across all blocks using separate service domains

VLAN Design & Traffic Optimization

- Voice VLAN, QoS planning, and Management/Server VLAN segmentation for VoIP prioritization and service separation

- Jumbo frames enabled for MPLS and service transport efficiency

Security & Access Control

- Inter-network security and traffic isolation

- RADIUS and Jump Server integration for centralized AAA authentication

IP Services

- Centralized DHCP server only; no DHCP services on routers

Block Routers and Loopback IPs (IPv4)

- LONDON_BLOCK (main block, connected to ISP): 10.0.0.1/32

- UK_BLOCK: 10.0.0.2/32

- NEPAL_BLOCK: 10.0.0.3/32

- HIMAL_BLOCK: 10.0.0.4/32

- BRIT_BLOCK: 10.0.0.5/32

- SKILL_BLOCK: 10.0.0.6/32

- ALUMNI_BLOCK: 10.0.0.7/32

- KUMARI_BLOCK: 10.0.0.8/32

Block Routers and Loopback IPs (IPv6)

- LONDON_BLOCK: fd4a:2b40:1000:1::1/128

- UK_BLOCK: fd4a:2b40:1000:1::2/128

- NEPAL_BLOCK: fd4a:2b40:1000:1::3/128

- HIMAL_BLOCK: fd4a:2b40:1000:1::4/128

- BRIT_BLOCK: fd4a:2b40:1000:1::5/128

- SKILL_BLOCK: fd4a:2b40:1000:1::6/128

- ALUMNI_BLOCK: fd4a:2b40:1000:1::7/128

- KUMARI_BLOCK: fd4a:2b40:1000:1::8/128

Point-to-Point Links from LONDON_BLOCK (IPv4 Private, /30)

- LONDON_BLOCK to UK_BLOCK: 10.0.4.0/30 (LONDON_BLOCK: 10.0.4.1, UK_BLOCK: 10.0.4.2)

- LONDON_BLOCK to NEPAL_BLOCK: 10.0.4.4/30 (LONDON_BLOCK: 10.0.4.5, NEPAL_BLOCK: 10.0.4.6)

- LONDON_BLOCK to HIMAL_BLOCK: 10.0.4.8/30 (LONDON_BLOCK: 10.0.4.9, HIMAL_BLOCK: 10.0.4.10)

- LONDON_BLOCK to BRIT_BLOCK: 10.0.4.12/30 (LONDON_BLOCK: 10.0.0.13, BRIT_BLOCK: 10.0.0.14)

- LONDON_BLOCK to SKILL_BLOCK: 10.0.4.16/30 (LONDON_BLOCK: 10.0.4.17, SKILL_BLOCK: 10.0.4.18)

- LONDON_BLOCK to KUMARI_BLOCK: 10.0.4.20/30 (LONDON_BLOCK: 10.0.4.21, KUMARI_BLOCK: 10.0.4.22)

- LONDON_BLOCK to ALUMNI_BLOCK: 10.0.4.24/30 (LONDON_BLOCK: 10.0.4.25, ALUMNI_BLOCK: 10.0.4.26)

Point-to-Point Links from LONDON_BLOCK (IPv6, /64)

- LONDON_BLOCK to UK_BLOCK: fd4a:2b40:1000:2::/64 (LONDON_BLOCK: fd4a:2b40:1000:2::1/64, UK_BLOCK: fd4a:2b40:1000:2::2/64)

- LONDON_BLOCK to NEPAL_BLOCK: fd4a:2b40:1000:2::/64 (LONDON_BLOCK: fd4a:2b40:1000:2::3/64, NEPAL_BLOCK: fd4a:2b40:1000:2::4/64)

- LONDON_BLOCK to HIMAL_BLOCK: fd4a:2b40:1000:2::/64 (LONDON_BLOCK: fd4a:2b40:1000:2::5/64, HIMAL_BLOCK: fd4a:2b40:1000:2::6/64)

- LONDON_BLOCK to BRIT_BLOCK: fd4a:2b40:1000:2::/64 (LONDON_BLOCK: fd4a:2b40:1000:2::7/64, BRIT_BLOCK: fd4a:2b40:1000:2::8/64)

- LONDON_BLOCK to SKILL_BLOCK: fd4a:2b40:1000:2::/64  (LONDON_BLOCK: fd4a:2b40:1000:2::9/64, SKILL_BLOCK: fd4a:2b40:1000:2::10/64)

- LONDON_BLOCK to KUMARI_BLOCK: fd4a:2b40:1000:2::/64 (LONDON_BLOCK: fd4a:2b40:1000:2::11/64, KUMARI_BLOCK: fd4a:2b40:1000:2::12/64)

- LONDON_BLOCK to ALUMNI_BLOCK: fd4a:2b40:1000:2::/64 (LONDON_BLOCK: fd4a:2b40:1000:2::13/64, ALUMNI_BLOCK: fd4a:2b40:1000:2::14/64)

Mesh Connections (Router-to-Router Private IPs, /30 subnets):

- UK_BLOCK to NEPAL_BLOCK: 10.0.4.28/30 (UK_BLOCK: 10.0.4.29, NEPAL_BLOCK: 10.0.4.30)

- UK_BLOCK to HIMAL_BLOCK: 10.0.4.32/30 (UK_BLOCK: 10.0.4.34, HIMAL_BLOCK: 10.0.4.33)

- NEPAL_BLOCK to BRIT_BLOCK: 10.0.4.36/30 (NEPAL_BLOCK: 10.0.4.37, BRIT_BLOCK: 10.0.4.38)

- HIMAL_BLOCK to SKILL_BLOCK: 10.0.4.40/30 (HIMAL_BLOCK: 10.0.4.42, SKILL_BLOCK: 10.0.4.41)

- BRIT_BLOCK to KUMARI_BLOCK: 10.0.4.44/30 (BRIT_BLOCK: 10.0.0.46, KUMARI_BLOCK: 10.0.0.45)

- SKILL_BLOCK to ALUMNI_BLOCK: 10.0.4.48/30 (SKILL_BLOCK: 10.0.4.50, ALUMNI_BLOCK: 10.0.4.49)

- ALUMNI_BLOCK to KUMARI_BLOCK: 10.0.4.52/30 (ALUMNI_BLOCK: 10.0.4.53, KUMARI_BLOCK: 10.0.4.54)

Mesh Connections (Router-to-Router IPv6, /64)

- UK_BLOCK to NEPAL_BLOCK: fd4a:2b40:1000:2::/64 (UK_BLOCK: fd4a:2b40:1000:2::15, NEPAL_BLOCK: fd4a:2b40:1000:2::16)

- UK_BLOCK to HIMAL_BLOCK: fd4a:2b40:1000:2::/64 (UK_BLOCK: fd4a:2b40:1000:2::17, HIMAL_BLOCK: fd4a:2b40:1000:2::18)

- NEPAL_BLOCK to BRIT_BLOCK: fd4a:2b40:1000:2::/64 (NEPAL_BLOCK: fd4a:2b40:1000:2::19, BRIT_BLOCK: fd4a:2b40:1000:2::20)

- HIMAL_BLOCK to SKILL_BLOCK: fd4a:2b40:1000:2::/64 (HIMAL_BLOCK: fd4a:2b40:1000:2::21, SKILL_BLOCK: fd4a:2b40:1000:2::22)

- BRIT_BLOCK to KUMARI_BLOCK: fd4a:2b40:1000:2::/64 (BRIT_BLOCK: fd4a:2b40:1000:2::23, KUMARI_BLOCK: fd4a:2b40:1000:2::24)

- SKILL_BLOCK to ALUMNI_BLOCK: fd4a:2b40:1000:2::/64 (SKILL_BLOCK: fd4a:2b40:1000:2::25, ALUMNI_BLOCK: fd4a:2b40:1000:2::26)

- ALUMNI_BLOCK to KUMARI_BLOCK: fd4a:2b40:1000:2::/64 (ALUMNI_BLOCK: fd4a:2b40:1000:2::27, KUMARI_BLOCK: fd4a:2b40:1000:2::28)

VLANs and End-Device Subnets (Provided by Centralized DHCP, Dual-Stack):

Note: VLAN IPv4 subnets use 10.10.0.0/16 to avoid conflict with the 10.0.0.x point-to-point core links.

VLAN 100 (MANAGEMENT):

LOOPBACK

- IPv4: 10.0.0.0/22 {Usable IPs: 1026}

- IPv6: fd4a:2b40:1000:1::1/64

Router or Switch Connection

- IPv4: 10.0.4.0/21 (GW 10.0.4.1) {Usable IPs: 2046}

- IPv6: fd4a:2b40:1000:2::1/64 (GW 2001:df4:2b40:2::1)

VLAN 200 (SERVER):

- IPv4: 10.0.8.0/23 (GW 10.10.8.1) {Usable IPs: 510}

- IPv6: fd4a:2b40:1000:3::1/64 (GW 2001:df4:2b40:3::1)

VLAN 300 (CCTV):

- IPv4: 10.0.16.0/20 (GW 10.10.16.1) {Usable IPs: 4094}

- IPv6: fd4a:2b40:1000:4::1/64 (GW 2001:df4:2b40:4::1)

VLAN 400 (VOICE / VOIP):

- IPv4: 10.0.32.0/19 (GW 10.10.32.1) {Usable IPs: 8190}

- IPv6: fd4a:2b40:1000:5::1/64 (GW 2001:df4:2b40:6::1)

VLAN 500 (TEACHER):

- IPv4: 10.0.64.0/21 (GW 10.10.64.1) {Usable IPs: 2046}

- IPv6: 2001:df4:2b40:6::1/64 (GW 2001:df4:2b40:7::1)

VLAN 600 (STAFF):

- IPv4: 10.0.72.0/20 (GW 10.10.72.1) {Usable IPs: 4094}

- IPv6: 2001:df4:2b40:7::1/64 (GW 2001:df4:2b40:8::1)

VLAN 700 (STUDENT):

- IPv4: 10.0.96.0/18 (GW 10.10.96.1) {Usable IPs: 16382}

- IPv6: 2001:df4:2b40:8::1/64 (GW 2001:df4:2b40:9::1)

Table of Contents

1.	Introduction	1

1.1.	Background of the Network Design	1

1.2.	Objectives of the New Implementation	1

1.3.	Design Requirements	2

1.4.	Network Services Overview	2

1.5.	Implementation Method	3

2.	Network Topology Overview	5

2.1.	Physical Topology Diagram	5

2.2.	Logical Topology Diagram	5

2.3.	Core, Distribution, and Access Roles	5

2.4.	Redundancy and High Availability Design	6

2.5.	Naming Convention for Devices	6

3.	Device Identity and Base Configuration	8

3.1.	Router Naming (Identity Configuration)	8

3.1.1.	LONDON-BLOCK	8

3.1.2.	UK-BLOCK	8

3.1.3.	NEPAL-BLOCK	8

3.1.4.	HIMAL-BLOCK	8

3.1.5.	BRIT-BLOCK	8

3.1.6.	SKILL-BLOCK	8

3.1.7.	ALUMNI-BLOCK	8

3.1.8.	KUMARI-BLOCK	8

3.2.	Management Access (Winbox/SSH)	9

3.2.1.	LONDON-BLOCK	9

3.2.2.	UK-BLOCK	9

3.2.3.	NEPAL-BLOCK	9

3.2.4.	HIMAL-BLOCK	9

3.2.5.	BRIT-BLOCK	9

3.2.6.	SKILL-BLOCK	10

3.2.7.	ALUMNI-BLOCK	10

3.2.8.	KUMARI-BLOCK	10

3.3.	Time, NTP and Logging Configuration	10

3.3.1.	LONDON-BLOCK	10

3.3.2.	UK-BLOCK	10

3.3.3.	NEPAL-BLOCK	10

3.3.4.	HIMAL-BLOCK	10

3.3.5.	BRIT-BLOCK	10

3.3.6.	SKILL-BLOCK	11

3.3.7.	ALUMNI-BLOCK	11

3.3.8.	KUMARI-BLOCK	11

4.	Interface Planning and Physical Connectivity	12

4.1.	Jumbo Frame and MTU Configuration	12

4.1.1.	Why MTU Matters in MPLS	12

4.1.2.	Core Router-to-Router MTU Configuration	12

4.1.3.	Trunk Port MTU Configuration (Router-Switch)	18

4.2.	Configure Bonding on Router	19

4.2.1.	LONDON-BLOCK	19

4.2.2.	UK-BLOCK	20

4.2.3.	NEPAL-BLOCK	20

4.2.4.	HIMAL-BLOCK	20

4.2.5.	BRIT-BLOCK	20

4.2.6.	SKILL-BLOCK	21

4.2.7.	ALUMNI-BLOCK	21

4.2.8.	KUMARI-BLOCK	21

4.3.	Interface Naming and Description	21

4.3.1.	LONDON-BLOCK	21

4.3.2.	UK-BLOCK	21

4.3.3.	NEPAL-BLOCK	21

4.3.4.	HIMAL-BLOCK	21

4.3.5.	BRIT-BLOCK	21

4.3.6.	SKILL-BLOCK	21

4.3.7.	ALUMNI-BLOCK	21

4.3.8.	KUMARI-BLOCK	22

4.4.	Router-to-Router Connectivity	22

4.4.1.	LONDON-BLOCK	22

4.4.2.	UK-BLOCK	22

4.4.3.	NEPAL-BLOCK	22

4.4.4.	HIMAL-BLOCK	22

4.4.5.	BRIT-BLOCK	22

4.4.6.	SKILL-BLOCK	22

4.4.7.	ALUMNI-BLOCK	22

4.4.8.	KUMARI-BLOCK	22

4.5.	Router-to-Switch Connectivity	22

4.5.1.	LONDON-BLOCK	22

4.5.2.	UK-BLOCK	22

4.5.3.	NEPAL-BLOCK	22

4.5.4.	HIMAL-BLOCK	22

4.5.5.	BRIT-BLOCK	22

4.5.6.	SKILL-BLOCK	22

4.5.7.	ALUMNI-BLOCK	22

4.5.8.	KUMARI-BLOCK	22

4.6.	Loop Protection and Unused Port Hardening	23

4.6.1.	LONDON-BLOCK	23

4.6.2.	UK-BLOCK	23

4.6.3.	NEPAL-BLOCK	23

4.6.4.	HIMAL-BLOCK	23

4.6.5.	BRIT-BLOCK	23

4.6.6.	SKILL-BLOCK	23

4.6.7.	ALUMNI-BLOCK	23

4.6.8.	KUMARI-BLOCK	23

5.	IP Addressing Plan	24

5.1.	IPv4 Interface Addressing	24

5.1.1.	LONDON-BLOCK	24

5.1.2.	UK-BLOCK	24

5.1.3.	NEPAL-BLOCK	24

5.1.4.	HIMAL-BLOCK	24

5.1.5.	BRIT-BLOCK	24

5.1.6.	SKILL-BLOCK	24

5.1.7.	ALUMNI-BLOCK	24

5.1.8.	KUMARI-BLOCK	24

5.2.	Loopback Address Allocation	24

5.2.1.	LONDON-BLOCK	24

5.2.2.	UK-BLOCK	24

5.2.3.	NEPAL-BLOCK	24

5.2.4.	HIMAL-BLOCK	24

5.2.5.	BRIT-BLOCK	24

5.2.6.	SKILL-BLOCK	24

5.2.7.	ALUMNI-BLOCK	24

5.2.8.	KUMARI-BLOCK	24

5.3.	Router-to-Router Point-to-Point /30 Links	25

5.3.1.	LONDON-BLOCK	25

5.3.2.	UK-BLOCK	25

5.3.3.	NEPAL-BLOCK	25

5.3.4.	HIMAL-BLOCK	25

5.3.5.	BRIT-BLOCK	25

5.3.6.	SKILL-BLOCK	25

5.3.7.	ALUMNI-BLOCK	25

5.3.8.	KUMARI-BLOCK	25

5.4.	IPv6 Dual-Stack Addressing	25

5.4.1.	LONDON-BLOCK	25

5.4.2.	UK-BLOCK	25

5.4.3.	NEPAL-BLOCK	25

5.4.4.	HIMAL-BLOCK	25

5.4.5.	BRIT-BLOCK	25

5.4.6.	SKILL-BLOCK	25

5.4.7.	ALUMNI-BLOCK	25

5.4.8.	KUMARI-BLOCK	25

6.	VLAN and Subnet Design	26

6.1.	VLAN Architecture Overview	26

6.2.	Student Network	26

6.3.	Staff Network	26

6.4.	Teacher Network	26

6.5.	CCTV Network	26

6.6.	Voice (VoIP) Network	26

6.7.	Guest Wi-Fi Network	26

6.8.	Management Network	26

6.9.	Server Network	26

7.	Interior Gateway Routing – OSPF & OSPFv3	27

7.1.	OSPF (IPv4) Configuration	27

7.1.1.	LONDON-BLOCK	27

7.1.2.	UK-BLOCK	27

7.1.3.	NEPAL-BLOCK	27

7.1.4.	HIMAL-BLOCK	27

7.1.5.	BRIT-BLOCK	27

7.1.6.	SKILL-BLOCK	27

7.1.7.	ALUMNI-BLOCK	27

7.1.8.	KUMARI-BLOCK	27

7.2.	OSPFv3 (IPv6) Configuration	27

7.2.1.	LONDON-BLOCK	27

7.2.2.	UK-BLOCK	27

7.2.3.	NEPAL-BLOCK	27

7.2.4.	HIMAL-BLOCK	27

7.2.5.	BRIT-BLOCK	27

7.2.6.	SKILL-BLOCK	27

7.2.7.	ALUMNI-BLOCK	27

7.2.8.	KUMARI-BLOCK	27

7.3.	OSPF Neighbor Verification	28

7.3.1.	LONDON-BLOCK	28

7.3.2.	UK-BLOCK	28

7.3.3.	NEPAL-BLOCK	28

7.3.4.	HIMAL-BLOCK	28

7.3.5.	BRIT-BLOCK	28

7.3.6.	SKILL-BLOCK	28

7.3.7.	ALUMNI-BLOCK	28

7.3.8.	KUMARI-BLOCK	28

7.4.	ECMP Load Balancing	28

7.4.1.	LONDON-BLOCK	28

7.4.2.	UK-BLOCK	28

7.4.3.	NEPAL-BLOCK	28

7.4.4.	HIMAL-BLOCK	28

7.4.5.	BRIT-BLOCK	28

7.4.6.	SKILL-BLOCK	28

7.4.7.	ALUMNI-BLOCK	28

7.4.8.	KUMARI-BLOCK	28

8.	MPLS Backbone Implementation	29

8.1.	Enable MPLS on Core Routers	29

8.2.	LDP Configuration	29

8.3.	MPLS Forwarding Verification	29

8.4.	Jumbo Frame / MPLS MTU Configuration	29

9.	VPLS Implementation	30

9.1.	VPLS Instance Creation	30

9.1.1.	LONDON-BLOCK	30

9.1.2.	UK-BLOCK	30

9.1.3.	NEPAL-BLOCK	30

9.1.4.	HIMAL-BLOCK	30

9.1.5.	BRIT-BLOCK	30

9.1.6.	SKILL-BLOCK	30

9.1.7.	ALUMNI-BLOCK	30

9.1.8.	KUMARI-BLOCK	30

9.2.	Multiple VPLS Domains (Student, Staff, CCTV, Voice, Management, Guest Wi-Fi)	30

9.2.1.	LONDON-BLOCK	30

9.2.2.	UK-BLOCK	30

9.2.3.	NEPAL-BLOCK	30

9.2.4.	HIMAL-BLOCK	30

9.2.5.	BRIT-BLOCK	30

9.2.6.	SKILL-BLOCK	30

9.2.7.	ALUMNI-BLOCK	30

9.2.8.	KUMARI-BLOCK	30

9.3.	Bridge and VLAN Mapping	31

9.3.1.	LONDON-BLOCK	31

9.3.2.	UK-BLOCK	31

9.3.3.	NEPAL-BLOCK	31

9.3.4.	HIMAL-BLOCK	31

9.3.5.	BRIT-BLOCK	31

9.3.6.	SKILL-BLOCK	31

9.3.7.	ALUMNI-BLOCK	31

9.3.8.	KUMARI-BLOCK	31

9.4.	VPLS Connectivity Verification	31

9.4.1.	LONDON-BLOCK	31

9.4.2.	UK-BLOCK	31

9.4.3.	NEPAL-BLOCK	31

9.4.4.	HIMAL-BLOCK	31

9.4.5.	BRIT-BLOCK	31

9.4.6.	SKILL-BLOCK	31

9.4.7.	ALUMNI-BLOCK	31

9.4.8.	KUMARI-BLOCK	31

10.	AAA Authentication (RADIUS & Jump Server)	32

10.1.	RADIUS Server Configuration	32

10.2.	Router Authentication via RADIUS	32

10.3.	Role-Based Access Control	32

10.4.	Jump Server Integration	32

10.5.	Accounting and Logging	32

11.	BGP Implementation	33

11.1.	BGP Design Overview	33

11.2.	eBGP Configuration	33

11.3.	iBGP Configuration	33

11.3.1.	LONDON-BLOCK	33

11.3.2.	UK-BLOCK	33

11.3.3.	NEPAL-BLOCK	33

11.3.4.	HIMAL-BLOCK	33

11.3.5.	BRIT-BLOCK	33

11.3.6.	SKILL-BLOCK	33

11.3.7.	ALUMNI-BLOCK	33

11.3.8.	KUMARI-BLOCK	33

11.4.	Route Advertisement and Filtering	33

11.5.	BGP Session Verification	33

12.	IPv6 Implementation	34

12.1.	Dual-Stack Routing Operation	34

12.2.	IPv6 Neighbor Discovery	34

12.3.	SLAAC / DHCPv6 Configuration	34

12.4.	IPv6 Connectivity Testing	34

13.	VLAN Segmentation & QoS	35

13.1.	Traffic Classification	35

13.2.	Voice VLAN Prioritization	35

13.3.	Guest Wi-Fi Bandwidth Control	35

13.4.	Queue Management	35

14.	Network Security and Traffic Isolation	36

14.1.	Inter-VLAN Access Control	36

14.2.	Management Network Protection	36

14.3.	All Network Isolation	36

15.	Monitoring and Verification	37

15.1.	Connectivity Testing	37

15.2.	Routing Table Verification	37

15.3.	MPLS Label Verification	37

15.4.	Failover Testing	37

16.	Troubleshooting	37

16.1.	OSPF Issues	37

16.2.	MPLS/LDP Issues	37

16.3.	VPLS Issues	37

16.4.	BGP Issues	37

16.5.	IPv6 Issues	37

17.	Conclusion and Future Work	37

17.1.	Project Summary	37

17.2.	Lessons Learned	37

17.3.	Future Improvements	37

18.	Project Outcomes	37

List of Figures

No table of figures entries found.

List of Tables

# Introduction

This project is an upgraded implementation of an MPLS and VPLS campus backbone network designed in GNS3 using MikroTik routers. The network is based on a multi-block academic campus scenario. The goal of this phase is to extend the basic MPLS backbone into a more realistic enterprise and service-provider style network.

The upgrade focuses on routing design, segmentation, authentication control, redundancy, and service separation. Instead of only providing connectivity, this version introduces structured network planning and better control mechanisms.

## Background of the Network Design

The original network was designed as a campus backbone using MPLS and VPLS technology. All block routers were connected using OSPF as the interior gateway protocol and MPLS with LDP for label switching. VLANs were extended across blocks using VPLS, and a centralized DHCP server provided IP addressing.

The previous implementation successfully provided Layer 2 connectivity across multiple academic blocks. However, it was mainly focused on basic routing and VLAN transport.

This upgrade builds on that foundation and adds more realistic enterprise features such as BGP simulation, IPv6 dual stack, AAA authentication, network segmentation, and traffic control.

## Objectives of the New Implementation

The main objectives of this upgrade are:

- To simulate a real enterprise/ISP-style campus backbone using MPLS architecture

- To implement both iBGP and eBGP for internal route distribution and external connectivity simulation

- To introduce IPv6 dual-stack routing alongside existing IPv4 infrastructure

- To segment and extend network services using multiple VPLS domains

- To implement RADIUS-based centralized authentication for secure administrative access

- To enhance network security through VLAN segmentation and traffic isolation

- To ensure high availability through redundancy and load balancing mechanisms

This phase focuses not only on connectivity, but also on scalable design, service control, and overall network stability.

## Design Requirements

The network design follows these key requirements:

- All block routers must participate in the MPLS backbone as core routers

- IS-IS must be used as the Interior Gateway Protocol (IGP) for both IPv4 and IPv6

- MPLS with LDP must be enabled on all core and interconnecting links

- iBGP must be used for internal route propagation across the backbone

- eBGP must be used to simulate ISP and external network connectivity

- Multiple VLANs must be extended across blocks using VPLS domains

- IPv4 and IPv6 must operate in dual-stack mode across the network

- Centralized AAA authentication must be implemented using RADIUS and a Jump Server

- Jumbo frames must be enabled to support MPLS and VPLS encapsulation overhead

- Network services must be logically segmented and isolated for enhanced security

The design must ensure redundancy, scalability, efficient traffic engineering, and structured network management.

## Network Services Overview

The upgraded network provides multiple logical services across the campus backbone:

- Student Network

- Staff Network

- Teacher Network

- CCTV Network

- Voice (VoIP) Network

- Guest Wi-Fi Network

- Management Network

- Server Network

Each service operates within its own VLAN and VPLS domain. This ensures traffic separation and better control. Voice traffic is prioritized using QoS mechanisms. Guest and management networks are isolated for security purposes.

The network also supports centralized DHCP, RADIUS authentication, and IPv6 services.

## Implementation Method

The implementation follows a structured and step-by-step configuration approach.

First, base router configurations are completed, including identity, loopback, and management access.

Second, IP addressing and VLAN segmentation are configured.

Third, interior routing using OSPF and OSPFv3 is implemented.

After routing stability is verified, MPLS and LDP are enabled on core links. VPLS instances are then created to transport VLAN traffic across blocks.

BGP is implemented to simulate external ISP connectivity.

Finally, AAA authentication, QoS policies, IPv6 services, and security controls are configured and tested.

All configurations are verified through connectivity testing, routing table inspection, MPLS label checking, and failover testing.

# Network Topology Overview

The campus network consists of multiple academic blocks connected through an MPLS core backbone. One main block acts as the central node, and other blocks connect using both direct and mesh links to provide redundancy.

The topology is designed to simulate a structured campus network with service-provider features.

## Physical Topology Diagram

The physical topology represents how routers and switches are interconnected in GNS3.

Each academic block contains a router connected to access switches. All block routers are interconnected through point-to-point core links.

The main block also connects to a simulated ISP for BGP implementation.

Mesh links between blocks provide alternative paths in case of failure.

## Logical Topology Diagram

The logical topology shows how routing and services operate across the network.

OSPF forms the interior routing domain across all routers. MPLS enables label switching across the backbone. VPLS extends Layer 2 VLANs across multiple blocks.

BGP operates between the main block and the simulated ISP.

IPv4 and IPv6 operate in dual-stack mode across all routing devices.

## Core, Distribution, and Access Roles

The network follows a simplified hierarchical structure:

Core Layer

The core layer consists of all MPLS-enabled routers forming the backbone. These routers handle routing, label switching, and traffic forwarding.

Access Layer

Each block connects to switches that provide VLAN access to end devices such as PCs, IP phones, CCTV cameras, and servers.

The core handles transport. The access layer handles user connectivity.

## Redundancy and High Availability Design

Redundancy is achieved using multiple point-to-point links between blocks.

OSPF supports Equal Cost Multi Path routing. This allows traffic to use multiple paths for load balancing.

If one core link fails, traffic automatically reroutes through alternate mesh links. This improves availability and reliability.

Failover testing confirms that connectivity is maintained during link failures.

## Naming Convention for Devices

Each router is named according to its campus block for easy identification.

Examples include:

LONDON-BLOCK

UK-BLOCK

NEPAL-BLOCK

HIMAL-BLOCK

BRIT-BLOCK

SKILL-BLOCK

ALUMNI-BLOCK

KUMARI-BLOCK

Loopback addresses are assigned based on the public IPv4 range defined in the design. Interface descriptions clearly identify link purposes such as core link, trunk link, or access connection.

This naming and addressing structure makes the network easier to manage and troubleshoot.

# Device Identity and Base Configuration

## Router Naming (Identity & loopback Configuration)

### LONDON-BLOCK

/system identity set name=LONDON-BLOCK /interface bridge add name=loopback comment="LONDON-BLOCK_10.0.0.1_LOOPBACK" /ip address add address= 10.0.0.1/32 interface=loopback comment="LONDON-BLOCK_10.0.0.1_LOOPBACK"

### UK-BLOCK

/system identity set name=UK-BLOCK /interface bridge add name=loopback comment="UK-BLOCK_10.0.0.2_LOOPBACK" /ip address add address= 10.0.0.2/32 interface=loopback comment="UK-BLOCK_10.0.0.2_LOOPBACK"

### NEPAL-BLOCK

/system identity set name=NEPAL -BLOCK /interface bridge add name=loopback comment="NEPAL-BLOCK_10.0.0.3_LOOPBACK" /ip address add address= 10.0.0.3/32 interface=loopback comment="NEPAL-BLOCK_10.0.0.3_LOOPBACK"

### HIMAL-BLOCK

/system identity set name=HIMAL-BLOCK /interface bridge add name=loopback comment="HIMAL-BLOCK_10.0.0.4_LOOPBACK" /ip address add address= 10.0.0.4/32 interface=loopback comment="HIMAL-BLOCK_10.0.0.4_LOOPBACK"

### BRIT-BLOCK

/system identity set name=BRIT-BLOCK /interface bridge add name=loopback comment="BRIT-BLOCK_10.0.0.5_LOOPBACK" /ip address add address= 10.0.0.5/32 interface=loopback comment="BRIT-BLOCK_10.0.0.5_LOOPBACK"

### SKILL-BLOCK

/system identity set name=SKILL-BLOCK /interface bridge add name=loopback comment="SKILL-BLOCK_10.0.0.6_LOOPBACK" /ip address add address= 10.0.0.6/32 interface=loopback comment="SKILL-BLOCK_10.0.0.6_LOOPBACK"

### ALUMNI-BLOCK

/system identity set name=ALUMNI-BLOCK /interface bridge add name=loopback comment="ALUMNI-BLOCK_10.0.0.7_LOOPBACK" /ip address add address= 10.0.0.7/32 interface=loopback comment="ALUMNI-BLOCK_10.0.0.7_LOOPBACK"

### KUMARI-BLOCK

/system identity set name=KUMARI-BLOCK /interface bridge add name=loopback comment="KUMARI-BLOCK_10.0.0.8_LOOPBACK" /ip address add address= 10.0.0.8/32 interface=loopback comment="KUMARI-BLOCK_10.0.0.8_LOOPBACK"

## Management Access (Winbox/SSH)

### LONDON-BLOCK

# Enable SSH on custom port (2222) /ip service enable ssh port=2222 # Enable ROMON (Router Management Over Network) /tool romon set enabled=yes # Restrict SSH access to a specific IP range /ip service set ssh address=10.10.0.0/21 # Disable unnecessary services /ip service disable www /ip service disable ftp # Add a firewall rule to restrict SSH access to a specific IP range /ip firewall filter add chain=input protocol=tcp dst-port=2222 src-address=10.10.0.0/21 action=accept /ip firewall filter add chain=input protocol=tcp dst-port=2222 action=drop # Disable unused services (HTTP, FTP, Telnet, etc.) /ip service disable telnet /ip service disable www /ip service disable ftp

### UK-BLOCK

/ip service disable telnet /ip service enable ssh port=2222 /tool/romon/set enabled=yes

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Time, NTP and Logging Configuration

### LONDON-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

### UK-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

### NEPAL-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

### HIMAL-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

### BRIT-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

### SKILL-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

### ALUMNI-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

### KUMARI-BLOCK

/system clock set time-zone-name=Asia/Kathmandu /system ntp client set enabled=yes /system ntp client servers add address=pool.ntp.org

# Interface Planning and Physical Connectivity

## Jumbo Frame and MTU Configuration

### Why MTU Matters in This MPLS Campus Network (MTU 9216)

Supports Large MPLS + VPLS Encapsulation

In this design, MPLS with LDP and multiple VPLS services (Student, Staff, CCTV, Voice, etc.) adds multiple labels and Ethernet encapsulation. With MTU 9216, the network can carry these large encapsulated frames without exceeding limits.

Result: Seamless Layer 2 extension across all 9 blocks without fragmentation

Ensures Reliable Dual-Stack (IPv4 + IPv6) Transport

The network runs dual-stack (IPv4 + IPv6) over MPLS. IPv6 packets are larger, and combined with MPLS labels, they increase frame size. Jumbo MTU ensures both protocols are transported efficiently.

Result: No packet drops for IPv6 and stable dual-stack performance

Prevents Packet Loss in Mesh + ECMP Core

Your topology uses mesh links and ECMP load balancing between blocks. If MTU were smaller, packets could be dropped on certain paths due to label stacking differences. MTU 9216 standardization across all links avoids this issue.

Result: Consistent forwarding and no blackholing across redundant paths

Optimizes High-Traffic VLAN Services (Voice, CCTV, Student)

Services like VoIP (VLAN 400), CCTV (VLAN 300), and Student network (VLAN 700) generate high traffic. Jumbo frames reduce overhead and improve throughput when carried over MPLS/VPLS backbone.

Result: Better performance, lower CPU usage on routers, and improved QoS efficiency

### Core Router-to-Router MTU Configuration

#### LONDON-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

#### UK-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

#### NEPAL-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

#### HIMAL-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

#### BRIT-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

#### SKILL-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

#### ALUMNI-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

#### KUMARI-BLOCK

/interface set ether1 mtu=9216 /interface set ether2 mtu=9216 /interface set ether3 mtu=9216 /interface set ether4 mtu=9216 /interface set ether5 mtu=9216 /interface set ether6 mtu=9216 /interface set ether7 mtu=9216 /interface set ether8 mtu=9216 /interface set ether9 mtu=9216 /interface set ether10 mtu=9216 /interface set ether11 mtu=9216 /interface set ether12 mtu=9216 /interface set ether13 mtu=9216 /interface set ether14 mtu=9216 /interface set ether15 mtu=9216 /interface set ether16 mtu=9216 /interface set ether17 mtu=9216 /interface set ether18 mtu=9216 /interface set ether19 mtu=9216 /interface set ether20 mtu=9216 /interface set ether21 mtu=9216 /interface set ether22 mtu=9216 /interface set ether23 mtu=9216 /interface set ether24 mtu=9216 /interface set ether25 mtu=9216 /interface set ether26 mtu=9216 /interface set ether27 mtu=9216 /interface set ether28 mtu=9216 /interface set ether29 mtu=9216 /interface set ether30 mtu=9216 /interface set ether31 mtu=9216 /interface set ether32 mtu=9216 SCRIPT :for i from=1 to=32 do={ /interface set ether$i mtu=9216 }

### Trunk Port MTU Configuration (Router-Switch)

#### LONDON-BLOCK

#### UK-BLOCK

#### NEPAL-BLOCK

#### HIMAL-BLOCK

#### BRIT-BLOCK

#### SKILL-BLOCK

#### ALUMNI-BLOCK

#### KUMARI-BLOCK

## Configure Bonding on Router

### LONDON-BLOCK

/interface bonding add name= LONDON-BLOCK_To_UK-BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="LONDON-BLOCK_To_UK-BLOCK" /interface bonding add name= LONDON-BLOCK_To_NEPAL_BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="LONDON-BLOCK_To_NEPAL-BLOCK" /interface bonding add name= LONDON-BLOCK_To_HIMAL_BLOCK mode=802.3ad slaves=ether7,ether8,ether9 transmit-hash-policy= layer-2 mtu= 9216 comment="LONDON-BLOCK_To_HIMAL-BLOCK" /interface bonding add name= LONDON-BLOCK_To_BRIT_BLOCK mode=802.3ad slaves=ether10,ether11,ether12 transmit-hash-policy= layer-2 mtu= 9216 comment="LONDON-BLOCK_To_BRIT-BLOCK" /interface bonding add name= LONDON-BLOCK_To_SKILL_BLOCK mode=802.3ad slaves=ether13,ether14,ether15 transmit-hash-policy= layer-2 mtu= 9216 comment="LONDON-BLOCK_To_SKILL-BLOCK" /interface bonding add name= LONDON-BLOCK_To_KUMARI-BLOCK mode=802.3ad slaves=ether16,ether17,ether18 transmit-hash-policy= layer-2 mtu= 9216 comment="LONDON-BLOCK_To_KUMARI-BLOCK" /interface bonding add name= LONDON-BLOCK_To_ALUMNI-BLOCK mode=802.3ad slaves=ether19,ether20,ether21 transmit-hash-policy= layer-2 mtu= 9216 comment="LONDON-BLOCK_To_ALUMNI-BLOCK"

### UK-BLOCK

/interface bonding add name= UK-BLOCK_To_LONDON-BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="UK-BLOCK_To_LONDON-BLOCK" /interface bonding add name= UK-BLOCK_To_HIMAL_BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="UK-BLOCK_To_HIMAL-BLOCK" /interface bonding add name= UK-BLOCK_To_NEPAL_BLOCK mode=802.3ad slaves=ether7,ether8,ether9 transmit-hash-policy= layer-2 mtu= 9216 comment="UK-BLOCK_To_NEPAL-BLOCK"

### NEPAL-BLOCK

/interface bonding add name= NEPAL-BLOCK_To_LONDON-BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="NEPAL-BLOCK_To_LONDON-BLOCK" /interface bonding add name= NEPAL-BLOCK_To_UK_BLOCK mode=802.3ad slaves=ether7,ether8,ether9 transmit-hash-policy= layer-2 mtu= 9216 comment="NEPAL-BLOCK_To_NEPAL-BLOCK" /interface bonding add name= NEPAL-BLOCK_To_BRIT_BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="NEPAL-BLOCK_To_BRIT-BLOCK"

### HIMAL-BLOCK

/interface bonding add name= HIMAL-BLOCK_To_LONDON-BLOCK mode=802.3ad slaves=ether7,ether8,ether9 transmit-hash-policy= layer-2 mtu= 9216 comment="HIMAL-BLOCK_To_LONDON-BLOCK" /interface bonding add name= HIMAL-BLOCK_To_UK_BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="HIMAL-BLOCK_To_UK-BLOCK" /interface bonding add name= HIMAL-BLOCK_To_SKILL_BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="HIMAL-BLOCK_To_SKILL-BLOCK"

### BRIT-BLOCK

/interface bonding add name= BRIT-BLOCK_To_LONDON-BLOCK mode=802.3ad slaves=ether10,ether11,ether12 transmit-hash-policy= layer-2 mtu= 9216 comment="BRIT-BLOCK_To_LONDON-BLOCK" /interface bonding add name= BRIT-BLOCK_To_KUMARI_BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="BRIT-BLOCK_To_KUMARI-BLOCK" /interface bonding add name= BRIT-BLOCK_To_NEPAL_BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="BRIT-BLOCK_To_NEPAL-BLOCK"

### SKILL-BLOCK

/interface bonding add name= SKILL-BLOCK_To_LONDON-BLOCK mode=802.3ad slaves=ether13,ether14,ether15 transmit-hash-policy= layer-2 mtu= 9216 comment="SKILL-BLOCK_To_LONDON-BLOCK" /interface bonding add name= SKILL-BLOCK_To_HIMAL_BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="SKILL-BLOCK_To_HIMAL-BLOCK" /interface bonding add name= SKILL-BLOCK_To_ALUMNI_BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="SKILL-BLOCK_To_ALUMNI-BLOCK"

### ALUMNI-BLOCK

/interface bonding add name= ALUMNI-BLOCK_To_LONDON-BLOCK mode=802.3ad slaves=ether16,ether17,ether18 transmit-hash-policy= layer-2 mtu= 9216 comment="ALUMNI-BLOCK_To_LONDON-BLOCK" /interface bonding add name= ALUMNI-BLOCK_To_KUMARI_BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="ALUMNI-BLOCK_To_KUMARI-BLOCK" /interface bonding add name= ALUMNI-BLOCK_To_SKILL_BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="ALUMNI-BLOCK_To_ALUMNI-BLOCK"

### KUMARI-BLOCK

/interface bonding add name= KUMARI-BLOCK_To_LONDON-BLOCK mode=802.3ad slaves=ether19,ether20,ether21 transmit-hash-policy= layer-2 mtu= 9216 comment="KUMARI-BLOCK_To_LONDON-BLOCK" /interface bonding add name= KUMARI-BLOCK_To_ALUMNI_BLOCK mode=802.3ad slaves=ether1,ether2,ether3 transmit-hash-policy= layer-2 mtu= 9216 comment="KUMARI-BLOCK_To_KUMARI-BLOCK" /interface bonding add name= KUMARI-BLOCK_To_BRIT_BLOCK mode=802.3ad slaves=ether4,ether5,ether6 transmit-hash-policy= layer-2 mtu= 9216 comment="KUMARI-BLOCK_To_BRIT-BLOCK"

## Interface Naming, Description & Loop Protection

### LONDON-BLOCK

/interface ethernet set ether1 loop-protect=on comment="NIC-Bonding Link ETHER1 From LONDON-BLOCK (10.0.4.1/30) to UK-BLOCK (10.0.4.2/30)" /interface ethernet set ether2 loop-protect=on comment="NIC-Bonding Link ETHER2 From LONDON-BLOCK (10.0.4.1/30) to UK-BLOCK (10.0.4.2/30)" /interface ethernet set ether3 loop-protect=on comment="NIC-Bonding Link ETHER3 From LONDON-BLOCK (10.0.4.1/30) to UK-BLOCK (10.0.4.2/30)" /interface ethernet set ether4 loop-protect=on comment="NIC-Bonding Link ETHER4 From LONDON-BLOCK (10.0.4.5/30) to NEPAL-BLOCK (10.0.4.6/30)" /interface ethernet set ether5 loop-protect=on comment="NIC-Bonding Link ETHER5 From LONDON-BLOCK (10.0.4.5/30) to NEPAL-BLOCK (10.0.4.6/30)" /interface ethernet set ether6 loop-protect=on comment="NIC-Bonding Link ETHER6 From LONDON-BLOCK (10.0.4.5/30) to NEPAL-BLOCK (10.0.4.6/30)" /interface ethernet set ether7 loop-protect=on comment="NIC-Bonding Link ETHER7 From LONDON-BLOCK (10.0.4.9/30) to HIMAL-BLOCK (10.0.4.10/30)" /interface ethernet set ether8 loop-protect=on comment="NIC-Bonding Link ETHER8 From LONDON-BLOCK (10.0.4.9/30) to HIMAL-BLOCK (10.0.4.10/30)" /interface ethernet set ether9 loop-protect=on comment="NIC-Bonding Link ETHER9 From LONDON-BLOCK (10.0.4.9/30) to HIMAL-BLOCK (10.0.4.10/30)" /interface ethernet set ether10 loop-protect=on comment="NIC-Bonding Link ETHER10 From LONDON-BLOCK (10.0.4.13/30) to BRIT-BLOCK (10.0.4.14/30)" /interface ethernet set ether11 loop-protect=on comment="NIC-Bonding Link ETHER11 From LONDON-BLOCK (10.0.4.13/30) to BRIT-BLOCK (10.0.4.14/30)" /interface ethernet set ether12 loop-protect=on comment="NIC-Bonding Link ETHER12 From LONDON-BLOCK (10.0.4.13/30) to BRIT-BLOCK (10.0.4.14/30)" /interface ethernet set ether13 loop-protect=on comment="NIC-Bonding Link ETHER13 From LONDON-BLOCK (10.0.4.17/30) to SKILL-BLOCK (10.0.4.18/30)" /interface ethernet set ether14 loop-protect=on comment="NIC-Bonding Link ETHER14 From LONDON-BLOCK (10.0.4.17/30) to SKILL-BLOCK (10.0.4.18/30)" /interface ethernet set ether15 loop-protect=on comment="NIC-Bonding Link ETHER15 From LONDON-BLOCK (10.0.4.17/30) to SKILL-BLOCK (10.0.4.18/30)" /interface ethernet set ether16 loop-protect=on comment="NIC-Bonding Link ETHER16 From LONDON-BLOCK (10.0.4.25/30) to ALUMNI-BLOCK (10.0.4.26/30)" /interface ethernet set ether17 loop-protect=on comment="NIC-Bonding Link ETHER17 From LONDON-BLOCK (10.0.4.25/30) to ALUMNI-BLOCK (10.0.4.26/30)" /interface ethernet set ether18 loop-protect=on comment="NIC-Bonding Link ETHER18 From LONDON-BLOCK (10.0.4.25/30) to ALUMNI-BLOCK (10.0.4.26/30)" /interface ethernet set ether16 loop-protect=on comment="NIC-Bonding Link ETHER16 From LONDON-BLOCK (10.0.4.21/30) to KUMARI-BLOCK (10.0.4.22/30)" /interface ethernet set ether17 loop-protect=on comment="NIC-Bonding Link ETHER17 From LONDON-BLOCK (10.0.4.21/30) to KUMARI-BLOCK (10.0.4.22/30)" /interface ethernet set ether18 loop-protect=on comment="NIC-Bonding Link ETHER18 From LONDON-BLOCK (10.0.4.21/30) to KUMARI-BLOCK (10.0.4.22/30)"

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Router-to-Switch Connectivity

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Loop Protection and Unused Port Hardening

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

# IP Addressing Plan

## IPv4 Interface Addressing

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Loopback Address Allocation

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Router-to-Router Point-to-Point /30 Links

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## IPv6 Dual-Stack Addressing

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

# VLAN and Subnet Design

## VLAN Architecture Overview

## Student Network

## Staff Network

## Teacher Network

## CCTV Network

## Voice (VoIP) Network

## Guest Wi-Fi Network

## Management Network

## Server Network

# Interior Gateway Routing – OSPF & OSPFv3

## OSPF (IPv4) Configuration

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## OSPFv3 (IPv6) Configuration

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## OSPF Neighbor Verification

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## ECMP Load Balancing

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

# MPLS Backbone Implementation

## Enable MPLS on Core Routers

## LDP Configuration

## MPLS Forwarding Verification

## Jumbo Frame / MPLS MTU Configuration

# VPLS Implementation

## VPLS Instance Creation

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Multiple VPLS Domains (Student, Staff, CCTV, Voice, Management, Guest Wi-Fi)

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Bridge and VLAN Mapping

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## VPLS Connectivity Verification

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

# AAA Authentication (RADIUS & Jump Server)

## RADIUS Server Configuration

## Router Authentication via RADIUS

## Role-Based Access Control

## Jump Server Integration

## Accounting and Logging

# BGP Implementation

## BGP Design Overview

## eBGP Configuration

## iBGP Configuration

### LONDON-BLOCK

### UK-BLOCK

### NEPAL-BLOCK

### HIMAL-BLOCK

### BRIT-BLOCK

### SKILL-BLOCK

### ALUMNI-BLOCK

### KUMARI-BLOCK

## Route Advertisement and Filtering

## BGP Session Verification

# IPv6 Implementation

## Dual-Stack Routing Operation

## IPv6 Neighbor Discovery

## SLAAC / DHCPv6 Configuration

## IPv6 Connectivity Testing

# VLAN Segmentation & QoS

## Traffic Classification

## Voice VLAN Prioritization

## Guest Wi-Fi Bandwidth Control

## Queue Management

# Network Security and Traffic Isolation

## Inter-VLAN Access Control

## Management Network Protection

## All Network Isolation

# Monitoring and Verification

## Connectivity Testing

## Routing Table Verification

## MPLS Label Verification

## Failover Testing

# Troubleshooting

## OSPF Issues

## MPLS/LDP Issues

## VPLS Issues

## BGP Issues

## IPv6 Issues

# Conclusion and Future Work

## Project Summary

## Lessons Learned

## Future Improvements

# Project Outcomes