z

1/1/2026

SUBASH SUBEDI

MPLS & VPLS Backbone Design based on Islington College Using GNS3 and MikroTik

MIKROTIK PROJECT

Scenario

Islington College operates a multi-block campus network consisting of 9 academic and administrative blocks. The college owns a public IPv4 address range (160.30.132.0/24), which must be used for router loopback interfaces within an MPLS backbone.

We are required to design, implement, and verify an MPLS-based core network using MikroTik routers in GNS3, providing VPLS-based Layer-2 connectivity across all blocks while using a centralized DHCP server.

The LONDON_BLOCK is the main block, connected to the Internet (ISP), and all other blocks connect to it directly and via additional mesh links for redundancy and load balancing.

Network Architecture and IP Addressing Plan:

Core Design

- All block routers act as MPLS core routers

- OSPF is the IGP across all routers

- MPLS with LDP must be enabled on all core and mesh links

- ECMP / Load balancing must be implemented

- VPLS must be used to extend VLANs across all blocks

- No DHCP service should run on any router

Use the following explicit IP addressing scheme. All loopback addresses are from the public IPv4 range 160.30.132.0/24 (160.30.132.0 to 160.30.132.255). Do not use any other IPs for loopbacks or links.

Block Routers and Loopback IPs

- LONDON_BLOCK (main block, connected to ISP): Loopback IP 160.30.132.1/32

- UK_BLOCK: Loopback IP 160.30.132.11/32

- NEPAL_BLOCK: Loopback IP 160.30.132.12/32

- HIMAL_BLOCK: Loopback IP 160.30.132.13/32

- BRIT_BLOCK: Loopback IP 160.30.132.14/32

- SKILL_BLOCK: Loopback IP 160.30.132.15/32

- KUMARI_BLOCK: Loopback IP 160.30.132.16/32

- ALUMNI_BLOCK: Loopback IP 160.30.132.17/32

Point-to-Point Links from LONDON_BLOCK (Private IPs, /30 subnets):

- LONDON_BLOCK to UK_BLOCK: 10.0.0.0/30 (LONDON_BLOCK: 10.0.0.1, UK_BLOCK: 10.0.0.2)

- LONDON_BLOCK to NEPAL_BLOCK: 10.0.0.4/30 (LONDON_BLOCK: 10.0.0.5, NEPAL_BLOCK: 10.0.0.6)

- LONDON_BLOCK to HIMAL_BLOCK: 10.0.0.8/30 (LONDON_BLOCK: 10.0.0.9, HIMAL_BLOCK: 10.0.0.10)

- LONDON_BLOCK to BRIT_BLOCK: 10.0.0.12/30 (LONDON_BLOCK: 10.0.0.13, BRIT_BLOCK: 10.0.0.14)

- LONDON_BLOCK to SKILL_BLOCK: 10.0.0.16/30 (LONDON_BLOCK: 10.0.0.17, SKILL_BLOCK: 10.0.0.18)

- LONDON_BLOCK to KUMARI_BLOCK: 10.0.0.20/30 (LONDON_BLOCK: 10.0.0.21, KUMARI_BLOCK: 10.0.0.22)

- LONDON_BLOCK to ALUMNI_BLOCK: 10.0.0.24/30 (LONDON_BLOCK: 10.0.0.25, ALUMNI_BLOCK: 10.0.0.26)

Mesh Connections (Router-to-Router Private IPs, /30 subnets for redundancy):

- UK_BLOCK to NEPAL_BLOCK: 10.0.0.28/30 (UK_BLOCK: 10.0.0.29, NEPAL_BLOCK: 10.0.0.30)

- UK_BLOCK to HIMAL_BLOCK: 10.0.0.32/30 (UK_BLOCK: 10.0.0.33, HIMAL_BLOCK: 10.0.0.34)

- UK_BLOCK to BRIT_BLOCK: 10.0.0.36/30 (UK_BLOCK: 10.0.0.37, BRIT_BLOCK: 10.0.0.38)

- HIMAL_BLOCK to SKILL_BLOCK: 10.0.0.40/30 (HIMAL_BLOCK: 10.0.0.41, SKILL_BLOCK: 10.0.0.42)

- SKILL_BLOCK to ALUMNI_BLOCK: 10.0.0.44/30 (SKILL_BLOCK: 10.0.0.45, ALUMNI_BLOCK: 10.0.0.46)

- ALUMNI_BLOCK to KUMARI_BLOCK: 10.0.0.48/30 (ALUMNI_BLOCK: 10.0.0.49, KUMARI_BLOCK: 10.0.0.50)

- KUMARI_BLOCK to BRIT_BLOCK: 10.0.0.52/30 (KUMARI_BLOCK: 10.0.0.53, BRIT_BLOCK: 10.0.0.54)

VLANs and End-Device Subnets (Provided by Centralized DHCP):

- VLAN 100 (STUDENT): 172.16.0.0/19 (capacity for 5,000 student IPs)

- VLAN 200 (TEACHER): 172.16.32.0/19 (capacity for 1,000 teacher IPs)

- VLAN 300 (STAFF): 172.16.36.0/21 (capacity for 2,000 staff IPs)

Table of Contents

1.	Set Identity, Loopback Address and RoMON to all Routers	1

1.1.	LONDON-BLOCK	1

1.2.	UK-BLOCK	2

1.3.	NEPAL-BLOCK	3

1.4.	HIMAL-BLOCK	4

1.5.	BRIT-BLOCK	5

1.6.	SKILL-BLOCK	6

1.7.	ALUMNI-BLOCK	7

1.8.	KUMARI-BLOCK	8

2.	Configuration IP in the Core Interface of all Routers	9

2.1.	LONDON-BLOCK	9

2.2.	UK-BLOCK	9

2.3.	NEPAL-BLOCK	9

2.4.	HIMAL-BLOCK	9

2.5.	BRIT-BLOCK	9

2.6.	SKILL-BLOCK	10

2.7.	ALUMNI-BLOCK	10

2.8.	KUMARI-BLOCK	10

3.	Configuration OSPF to all Core Routers	11

3.1.	LONDON-BLOCK	11

3.2.	UK-BLOCK	11

3.3.	NEPAL-BLOCK	11

3.4.	HIMAL-BLOCK	11

3.5.	BRIT-BLOCK	11

3.6.	SKILL-BLOCK	11

3.7.	ALUMNI-BLOCK	11

3.8.	KUMARI-BLOCK	11

4.	Configuration MPLS to all Core Routers	13

4.1.	LONDON-BLOCK	13

4.2.	UK-BLOCK	13

4.3.	NEPAL-BLOCK	13

4.4.	HIMAL-BLOCK	13

4.5.	BRIT-BLOCK	13

4.6.	SKILL-BLOCK	13

4.7.	ALUMNI-BLOCK	13

4.8.	KUMARI-BLOCK	13

5.	Configuration LDP on Interfaces of Core Routers	14

5.1.	LONDON-BLOCK	14

5.2.	UK-BLOCK	14

5.3.	NEPAL-BLOCK	14

5.4.	HIMAL-BLOCK	14

5.5.	BRIT-BLOCK	14

5.6.	SKILL-BLOCK	14

5.7.	ALUMNI-BLOCK	14

5.8.	KUMARI-BLOCK	14

6.	Set MPLS MTU (Crucial for VPLS to prevent fragmentation)	16

6.1.	LONDON-BLOCK	16

6.2.	UK-BLOCK	16

6.3.	NEPAL-BLOCK	16

6.4.	HIMAL-BLOCK	16

6.5.	BRIT-BLOCK	16

6.6.	SKILL-BLOCK	16

6.7.	ALUMNI-BLOCK	16

6.8.	KUMARI-BLOCK	16

7.	VPLS Setup (Points to Points) Routers	17

7.1.	LONDON-BLOCK	17

7.2.	UK-BLOCK	17

7.3.	NEPAL-BLOCK	17

7.4.	HIMAL-BLOCK	17

7.5.	BRIT-BLOCK	17

7.6.	SKILL-BLOCK	17

7.7.	ALUMNI-BLOCK	17

7.8.	KUMARI-BLOCK	17

8.	VPLS Configuration for VLAN Transport	19

8.1.	LONDON-BLOCK	19

8.2.	UK-BLOCK	19

8.3.	NEPAL-BLOCK	19

8.4.	HIMAL-BLOCK	19

8.5.	BRIT-BLOCK	19

8.6.	SKILL-BLOCK	19

8.7.	ALUMNI-BLOCK	19

8.8.	KUMARI-BLOCK	19

9.	End-to-End Verification and Traffic Analysis	20

9.1.	LONDON-BLOCK	20

9.2.	UK-BLOCK	20

9.3.	NEPAL-BLOCK	20

9.4.	HIMAL-BLOCK	20

9.5.	BRIT-BLOCK	20

9.6.	SKILL-BLOCK	20

9.7.	ALUMNI-BLOCK	20

9.8.	KUMARI-BLOCK	20

10.	Conclusion and Future Enhancements	21

11.	a	21

List of Figures

Figure 1: Set Identity, Loopback Address and RoMoN of LONDON_BLOCK Router Through CMD	1

Figure 2: Set Identity, Loopback Address and RoMoN of LONDON_BLOCK Router Through WINBOX	1

Figure 3: Set Identity, Loopback Address and RoMoN of UK-BLOCK Router Through CMD	2

Figure 4: Set Identity, Loopback Address and RoMoN of UK-BLOCK Router Through WINBOX	2

Figure 5: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through CMD	3

Figure 6: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through WINBOX	3

Figure 7: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through CMD	4

Figure 8: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through WINBOX	4

Figure 9: Set Identity, Loopback Address and RoMoN of BRIT-BLOCK Router Through CMD	5

Figure 10: Set Identity, Loopback Address and RoMoN of BRIT-BLOCK Router Through WINBOX	5

Figure 11: Set Identity, Loopback Address and RoMoN of SKILL-BLOCK Router Through CMD	6

Figure 12: Set Identity, Loopback Address and RoMoN of SKILL-BLOCK Router Through WINBOX	6

Figure 13: Set Identity, Loopback Address and RoMoN of ALUMNI-BLOCK Router Through CMD	7

Figure 14: Set Identity, Loopback Address and RoMoN of ALUMNI-BLOCK Router Through WINBOX	7

Figure 15: Set Identity, Loopback Address and RoMoN of KUMARI-BLOCK Router Through CMD	8

Figure 16: Set Identity, Loopback Address and RoMoN of KUMARI-BLOCK Router Through WINBOX	8

List of Tables

# Set Identity, Loopback Address and RoMON to all Routers

## LONDON-BLOCK

CMD

/system identity set name=LONDON-BLOCK /interface bridge add name=loopback comment="LONDON-BLOCK_160.30.132.1_LOOPBACK" /ip address add address=160.30.132.1/32 interface=loopback comment="LONDON-BLOCK_160.30.132.1_LOOPBACK" /tool/romon/set enabled=yes

Figure 1: Set Identity, Loopback Address and RoMoN of LONDON_BLOCK Router Through CMD

Figure 2: Set Identity, Loopback Address and RoMoN of LONDON_BLOCK Router Through WINBOX

## UK-BLOCK

CMD

/system identity set name= UK-BLOCK /interface bridge add name=loopback comment=" UK-BLOCK_160.30.132.11_LOOPBACK" /ip address add address=160.30.132.11/32 interface=loopback comment=" UK-BLOCK_160.30.132.11_LOOPBACK" /tool/romon/set enabled=yes

Figure 3: Set Identity, Loopback Address and RoMoN of UK-BLOCK Router Through CMD

Figure 4: Set Identity, Loopback Address and RoMoN of UK-BLOCK Router Through WINBOX

## NEPAL-BLOCK

CMD

/system identity set name=NEPAL-BLOCK /interface bridge add name=loopback comment="NEPAL-BLOCK_160.30.132.12_LOOPBACK" /ip address add address=160.30.132.12/32 interface=loopback comment="NEPAL-BLOCK_160.30.132.12_LOOPBACK" /tool/romon/set enabled=yes

Figure 5: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through CMD

Figure 6: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through WINBOX

## HIMAL-BLOCK

CMD

/system identity set name=HIMAL-BLOCK /interface bridge add name=loopback comment="HIMAL-BLOCK_160.30.132.13_LOOPBACK" /ip address add address=160.30.132.13/32 interface=loopback comment="HIMAL-BLOCK_160.30.132.13_LOOPBACK" /tool/romon/set enabled=yes

Figure 7: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through CMD

Figure 8: Set Identity, Loopback Address and RoMoN of NEPAL-BLOCK Router Through WINBOX

## BRIT-BLOCK

CMD

/system identity set name=BRIT-BLOCK /interface bridge add name=loopback comment="BRIT-BLOCK_160.30.132.14_LOOPBACK" /ip address add address=160.30.132.14/32 interface=loopback comment="BRIT-BLOCK_160.30.132.14_LOOPBACK" /tool/romon/set enabled=yes

Figure 9: Set Identity, Loopback Address and RoMoN of BRIT-BLOCK Router Through CMD

Figure 10: Set Identity, Loopback Address and RoMoN of BRIT-BLOCK Router Through WINBOX

## SKILL-BLOCK

CMD

/system identity set name=SKILL-BLOCK /interface bridge add name=loopback comment="SKILL-BLOCK_160.30.132.14_LOOPBACK" /ip address add address=160.30.132.14/32 interface=loopback comment="SKILL-BLOCK_160.30.132.14_LOOPBACK" /tool/romon/set enabled=yes

Figure 11: Set Identity, Loopback Address and RoMoN of SKILL-BLOCK Router Through CMD

Figure 12: Set Identity, Loopback Address and RoMoN of SKILL-BLOCK Router Through WINBOX

## ALUMNI-BLOCK

CMD

/system identity set name=ALUMNI-BLOCK /interface bridge add name=loopback comment="ALUMNI-BLOCK_160.30.132.15_LOOPBACK" /ip address add address=160.30.132.15/32 interface=loopback comment="ALUMNI-BLOCK_160.30.132.15_LOOPBACK" /tool/romon/set enabled=yes

Figure 13: Set Identity, Loopback Address and RoMoN of ALUMNI-BLOCK Router Through CMD

Figure 14: Set Identity, Loopback Address and RoMoN of ALUMNI-BLOCK Router Through WINBOX

## KUMARI-BLOCK

CMD

/system identity set name=KUMARI-BLOCK /interface bridge add name=loopback comment="KUMARI-BLOCK_160.30.132.16_LOOPBACK" /ip address add address=160.30.132.16/32 interface=loopback comment="KUMARI-BLOCK_160.30.132.16_LOOPBACK" /tool/romon/set enabled=yes

Figure 15: Set Identity, Loopback Address and RoMoN of KUMARI-BLOCK Router Through CMD

Figure 16: Set Identity, Loopback Address and RoMoN of KUMARI-BLOCK Router Through WINBOX

# Configuration IP in the Core Interface of all Routers

## LONDON-BLOCK

CMD

/ip address add address=10.0.0.1/30 interface=ether2 comment="LINK FROM LONDON-BLOCK_10.0.0.1/30 TO UK-BLOCK_10.0.0.2/30" /ip address add address=10.0.0.5/30 interface=ether3 comment="LINK FROM LONDON-BLOCK_10.0.0.5/30 TO NEPAL-BLOCK_10.0.0.6/30" /ip address add address=10.0.0.9/30 interface=ether4 comment="LINK FROM LONDON-BLOCK_10.0.0.9/30 TO BRIT-BLOCK_10.0.0.10/30" /ip address add address=10.0.0.13/30 interface=ether5 comment="LINK FROM LONDON-BLOCK_10.0.0.13/30 TO HIMAL-BLOCK_10.0.0.14/30" /ip address add address=10.0.0.17/30 interface=ether6 comment="LINK FROM LONDON-BLOCK_10.0.0.17/30 TO SKILL-BLOCK_10.0.0.18/30" /ip address add address=10.0.0.21/30 interface=ether7 comment="LINK FROM LONDON-BLOCK_10.0.0.21/30 TO ALUMNI-BLOCK_10.0.0.22/30" /ip address add address=10.0.0.25/30 interface=ether8 comment="LINK FROM LONDON-BLOCK_10.0.0.25/30 TO KUMARI-BLOCK_10.0.0.26/30"

## UK-BLOCK

CMD

## NEPAL-BLOCK

CMD

## HIMAL-BLOCK

CMD

## BRIT-BLOCK

CMD

## SKILL-BLOCK

CMD

## ALUMNI-BLOCK

CMD

## KUMARI-BLOCK

CMD

# Configuration OSPF to all Core Routers

## LONDON-BLOCK

## UK-BLOCK

## NEPAL-BLOCK

## HIMAL-BLOCK

## BRIT-BLOCK

## SKILL-BLOCK

## ALUMNI-BLOCK

## KUMARI-BLOCK

# Configuration MPLS to all Core Routers

## LONDON-BLOCK

## UK-BLOCK

## NEPAL-BLOCK

## HIMAL-BLOCK

## BRIT-BLOCK

## SKILL-BLOCK

## ALUMNI-BLOCK

## KUMARI-BLOCK

# Configuration LDP on Interfaces of Core Routers

## LONDON-BLOCK

## UK-BLOCK

## NEPAL-BLOCK

## HIMAL-BLOCK

## BRIT-BLOCK

## SKILL-BLOCK

## ALUMNI-BLOCK

## KUMARI-BLOCK

# Set MPLS MTU (Crucial for VPLS to prevent fragmentation)

## LONDON-BLOCK

## UK-BLOCK

## NEPAL-BLOCK

## HIMAL-BLOCK

## BRIT-BLOCK

## SKILL-BLOCK

## ALUMNI-BLOCK

## KUMARI-BLOCK

# VPLS Setup (Points to Points) Routers

## LONDON-BLOCK

## UK-BLOCK

## NEPAL-BLOCK

## HIMAL-BLOCK

## BRIT-BLOCK

## SKILL-BLOCK

## ALUMNI-BLOCK

## KUMARI-BLOCK

# VPLS Configuration for VLAN Transport

## LONDON-BLOCK

## UK-BLOCK

## NEPAL-BLOCK

## HIMAL-BLOCK

## BRIT-BLOCK

## SKILL-BLOCK

## ALUMNI-BLOCK

## KUMARI-BLOCK

# End-to-End Verification and Traffic Analysis

## LONDON-BLOCK

## UK-BLOCK

## NEPAL-BLOCK

## HIMAL-BLOCK

## BRIT-BLOCK

## SKILL-BLOCK

## ALUMNI-BLOCK

## KUMARI-BLOCK

# A

# Conclusion and Future Enhancements

# a