2025

Subash Subedi

VLANs: From Theory to Packet-Level Analysis with GNS3 and Wireshark

From Theory to Packet-Level Reality Perfect for Real-World Engineers

Abstract

This hands-on guide provides a complete understanding of IEEE 802.1Q VLANs using a practical GNS3 topology with one Cisco Layer 2 switch and seven PCs. The document explains VLAN theory, switch configuration, tagging and untagging behavior, and packet-level verification using Wireshark.

A Local SPAN (Switched Port Analyzer) session is used to mirror VLAN traffic to a dedicated monitoring port. This allows accurate observation of 802.1Q header behavior on access ports, trunk ports, and native VLAN traffic.

By completing this lab, learners gain both theoretical and practical understanding required for CCNA and real-world network troubleshooting.

Table of Contents

1.	Introduction	4

1.1.	What is a VLAN?	4

1.2.	Why VLANs Are Important	4

1.3.	Benefits of VLANs in Modern Networks	5

2.	VLAN Core Concepts	5

2.1.	Broadcast Domains & Network Segmentation	5

2.2.	Tagged vs Untagged Traffic	6

2.3.	IEEE 802.1Q Standard Overview	6

2.4.	The Native VLAN Concept	6

2.5.	VLAN IDs and Ranges	7

3.	VLAN Types	7

3.1.	Static VLANs (Port-Based)	7

3.2.	Dynamic VLANs	7

3.3.	Voice VLAN (Auto & Manual)	7

3.4.	Management VLAN & SVI	8

3.5.	Default VLAN (VLAN 1)	8

3.6.	Private VLANs	8

4.	Switch Port Modes	9

4.1.	Access Port	9

4.2.	Trunk Port (ISLCisco & IEEE 802.1Q)	9

4.3.	Dynamic Auto / Dynamic Desirable (DTP Modes Table)	9

4.4.	How Switches Decide: Tagging vs Untagging Rules (Decision Matrix)	10

5.	Hands-On Lab: VLAN Implementation in GNS3	11

5.1.	Lab Objectives	11

5.2.	Lab Topology Overview	11

5.3.	Devices and Software Used	11

5.4.	VLAN & IP Allocation Table	11

5.5.	Step-by-Step Switch Configuration	12

5.5.1.	Creating VLANs	12

5.5.2.	Configuring Access Ports	13

6.	SPAN (Switched Port Analyzer) Configuration	17

6.1.	Why Use SPAN in VLAN Labs	17

6.2.	Updated Topology with Monitoring Port	17

6.3.	Complete SPAN Configuration	17

6.4.	Verifying SPAN Session	18

6.5.	What You Should See in Wireshark	18

6.6.	Tag Behavior Summary Table	18

7.	Traffic Capture & VLAN Tag Analysis	19

7.1.	Access-Port Traffic --> Untagged	19

7.2.	Trunk Traffic --> Tagged (VLAN 10)	19

7.3.	802.1Q Header Breakdown in Wireshark	19

7.4.	Native VLAN Traffic --> Untagged on Trunk	19

7.5.	Sample Packet Capture	20

8.	Key Observations & Learning Outcomes	20

9.	Best Practices & Security Recommendations	20

10.	Troubleshooting VLAN Issues	20

10.1.	Top 5 VLAN Problems on the Exam	20

10.2.	Native VLAN Mismatch Symptoms	21

10.3.	Must-Know Commands + Expected Output	21

11.	Conclusion	21

# Introduction

## What is a VLAN?

A Virtual LAN (VLAN) is a logical grouping of devices within a switched network that allows them to communicate as if they were on the same physical segment, even if they are connected to different switches or located on different floors.

VLANs rely on logical connections rather than physical ones. Devices in the same VLAN share a broadcast domain, meaning that unicast, broadcast, and multicast traffic are restricted to that VLAN. Devices in different VLANs cannot communicate directly at Layer 2; communication between VLANs requires a Layer 3 device, such as a router.

Example: In a three-floor office, IT, HR, and Sales departments may each have their own VLAN (VLAN 2, 3, and 4). These VLANs span all floors, allowing devices to communicate logically while remaining physically connected to different switches.

## Why VLANs Are Important

VLANs are essential because they:

Segment networks logically by function, department, or application, regardless of physical location.

Reduce broadcast domains, preventing unnecessary traffic from reaching all devices and improving performance.

Contain broadcast traffic within each VLAN, improving efficiency and scalability.

Enhance security and access control by isolating sensitive users or departments (e.g., HR, IT) from general users (e.g., Guests, Students).

Enable flexible network design, allowing devices to be moved or added without rewiring or creating new physical networks.

## Benefits of VLANs in Modern Networks

The main advantages of VLAN implementation include:

Improved Performance

Smaller broadcast domains reduce unnecessary traffic and increase network efficiency.

Enhanced Security

Logical isolation prevents unauthorized access between VLANs, protecting sensitive departments.

Better Organization and Flexibility

Devices can be grouped by role or function rather than physical location, simplifying moves and expansions.

Simplified Network Management

Supports hierarchical IP addressing, making administration and troubleshooting easier.

Cost Efficiency

Reduces the need for multiple physical networks and equipment while maintaining logical separation.

Policy Enforcement

Access control and traffic rules can be applied per VLAN, such as restricting Guest VLANs to internet-only access.

# VLAN Core Concepts

## Broadcast Domains & Network Segmentation

A broadcast domain is a set of devices that receive a Layer 2 broadcast frame.

Without VLANs: the entire switched network is one broadcast domain - any broadcast is sent to all devices, causing unnecessary traffic.

With VLANs: each VLAN forms a separate broadcast domain. Broadcasts are limited to devices in the same VLAN, even across different switches.

Result: smaller broadcast domains → improved network performance, scalability, and security.

## Tagged vs Untagged Traffic

Untagged traffic: Standard Ethernet frames without VLAN information (from PCs or standard devices).

Tagged traffic: Frames with a 4-byte 802.1Q tag inserted between Source MAC and Type/Length fields, carrying the VLAN ID (VID).

Access ports: Send/receive untagged frames (single VLAN).

Trunk ports: Carry tagged frames for multiple VLANs.

Exception: Traffic in the native VLAN is sent untagged, even on trunks.

## IEEE 802.1Q Standard Overview

IEEE 802.1Q is the industry-standard protocol for VLAN tagging on Ethernet networks.

It inserts a 4-byte tag into the original Ethernet frame header containing:

TPID (Tag Protocol Identifier): 2 bytes, value 0x8100

Priority (PRI): 3 bits for Class of Service (QoS)

CFI: 1 bit (legacy Token Ring support)

VID (VLAN ID): 12 bits, supporting up to 4096 VLANs (0–4095)

After tagging, the switch recalculates and updates the Frame Check Sequence (FCS). 802.1Q enables trunk links to transport traffic from multiple VLANs between switches and other devices.s.

## The Native VLAN Concept

Each 802.1Q trunk has a native VLAN (default = VLAN 1).

Untagged frames received on a trunk are assigned to the native VLAN.

Frames for the native VLAN are sent untagged across the trunk.

Important points:

Cisco switches drop incoming tagged frames that match the native VLAN ID.

Control protocols (CDP, STP, VTP, DTP) typically use the native VLAN and are sent untagged.

Security best practice: Change the native VLAN to an unused VLAN (e.g., 999) and do not carry user traffic on it.

Native VLAN must match on both ends of the trunk; mismatch causes errors and warnings.

## VLAN IDs and Ranges

Normal VLANs: 1–1005 (stored in vlan.dat, supported on all Cisco switches)

Extended VLANs: 1006–4094 (supported on modern switches in VTP transparent mode)

Reserved IDs: 0 and 4095 cannot be used

Total VLANs possible with 802.1Q: 4096 (12-bit VID field)

# VLAN Types

## Static VLANs (Port-Based)

The most common and recommended method.

Ports are manually assigned to a specific VLAN using switchport access vlan X command.

Reliable, secure, and easy to understand - this is the primary method tested in CCNA.

## Dynamic VLANs

VLAN assignment is based on the device’s MAC address using a VLAN Membership Policy Server (VMPS).

Rarely used today and officially deprecated by Cisco - no longer part of CCNA but may appear in CCNP ENCOR.

## Voice VLAN (Auto & Manual)

Allows a single switch port to support both a PC (data) and an IP phone (voice).

The port is configured with one data VLAN and one voice VLAN.

Cisco IP phones use CDP or LLDP to learn the voice VLAN and tag voice traffic accordingly.

Configuration example:

switchport mode access

switchport access vlan 20

switchport voice vlan 150

## Management VLAN & SVI

A dedicated VLAN used for switch management (SSH, Telnet, SNMP, etc.).

Should not be VLAN 1 for security reasons.

A Switch Virtual Interface (SVI) is created using:

interface vlan 99

ip address 192.168.99.10 255.255.255.0

This provides Layer 3 access to the switch.

## Default VLAN (VLAN 1)

All switch ports belong to VLAN 1 by default.

VLAN 1 cannot be deleted or renamed.

Carries all control plane traffic (CDP, STP, VTP, DTP, etc.).

Best security practice:

Do not use VLAN 1 for user or management traffic

Change the native VLAN on trunks

Create separate management and user VLANs

## Private VLANs

Advanced feature for further Layer 2 isolation within a single VLAN.

Used in service provider or multi-tenant environments.

Types of ports:

Isolated: Can only communicate with promiscuous ports

Community: Can communicate with other members of the same community and promiscuous ports

Promiscuous: Typically connected to router/firewall

# Switch Port Modes

## Access Port

Belongs to exactly one VLAN.

Sends and receives only untagged frames.

Used for end devices (PCs, printers, IP phones).

Configuration:

switchport mode access

switchport access vlan X

Strongly recommended to explicitly configure mode access (security).

## Trunk Port (ISLCisco & IEEE 802.1Q)

Carries traffic for multiple VLANs.

Uses tagging to identify VLAN membership.

ISL (Cisco proprietary) - deprecated.

802.1Q (IEEE standard) - current standard used everywhere.

Configuration:

switchport mode trunk

switchport trunk native vlan 999

switchport trunk allowed vlan 10,20,30

## Dynamic Auto / Dynamic Desirable (DTP Modes Table)

Dynamic Trunking Protocol (DTP) negotiates trunking:

dynamic desirable: Actively tries to form a trunk - trunks with trunk, desirable, or auto

dynamic auto: Passively waits - trunks only if partner is trunk or desirable

trunk: Permanently trunking

access: Permanently access (no trunking)

Best practice: Disable DTP with switchport nonegotiate

## How Switches Decide: Tagging vs Untagging Rules (Decision Matrix)

When a frame enters a port:

Access port ---> untagged frame assigned to configured access VLAN

Trunk port ---> tagged frame forwarded based on VID

Trunk port ---> untagged frame assigned to native VLAN

When a frame exits a port:

Access port ---> always sent untagged

Trunk port ---> tagged (except native VLAN traffic → sent untagged)

| Port Type | Incoming Frame | Switch Action | Outgoing Frame |
| --- | --- | --- | --- |
| Access | Any | Assign to access VLAN | Untagged |
| Trunk | Tagged | Forward by VLAN ID | Tagged |
| Trunk | Untagged | Assigned to native VLAN | Untagged |
| Trunk | Native VLAN | Forward | Untagged |
| Trunk | Other VLAN | Forward | Tagged |

# Hands-On Lab: VLAN Implementation in GNS3

## Lab Objectives

Create VLAN 10 (IT), VLAN 20 (Finance), VLAN 30 (Sales)

Assign ports to correct VLANs

Configure Management VLAN

Use SPAN to capture frames and verify tagging behavior

## Lab Topology Overview

One Cisco L2 switch

Six PCs across three VLANs

One monitoring PC for SPAN

## Devices and Software Used

GNS3

Cisco vIOS-L2

Seven VPCS nodes

Wireshark

## VLAN & IP Allocation Table

| DEPARTMENT | VLAN | Name | IP Address |
| --- | --- | --- | --- |
| IT Department | VLAN 10 | PC1-VLAN--10 | 192.168.1.2 |
|  |  | PC2-VLAN--10 | 192.168.1.3 |
| Finance Department | VLAN 20 | ` PC3-VLAN--20 | 192.168.1.4 |
|  |  | PC4-VLAN--20 | 192.168.1.5 |
| Sales Department | VLAN 30 | PC5-VLAN--30 | 192.168.1.6 |
|  |  | PC6-VLAN--30 | 192.168.1.7 |
| SPAN (Switched Port Analyzer) Management | VLAN 99 | Switch SVI | 192.168.1.1 |

# Step-by-Step Switch Configuration

### Creating VLANs

enable configure terminal vlan 10 name IT Department exit vlan 20 name Finance Department exit vlan 30 name Sales Department exit vlan 99 name Management exit do show vlan do wr

### Configuring Access Ports

VLAN 10

enable configure terminal interface range GigabitEthernet0/0 - 3 switchport mode access switchport access vlan 10 description THIS INTERFACE BELONG TO IT DEPARTMENT no shutdown exit do show vlan ¦ include IT

VLAN 20

enable configure terminal interface range GigabitEthernet1/0 - 3 switchport mode access switchport access vlan 20 description THIS INTERFACE BELONG TO FIANANCE DEPARTMENT no shutdown exit do show vlan ¦ include Fi

VLAN 30

enable configure terminal interface range GigabitEthernet2/0 - 3 switchport mode access switchport access vlan 30 description THIS INTERFACE BELONG TO SALES DEPARTMENT no shutdown exit do show vlan ¦ include Sales

VLAN 99

enable configure terminal interface range GigabitEthernet3/0 - 3 switchport mode access switchport access vlan 99 description THIS INTERFACE BELONG TO MANAGEMENT SVI no shutdown exit do show vlan ¦ include Management

# SPAN (Switched Port Analyzer) Configuration

## Why Use SPAN in VLAN Labs

To see the truth - prove that access ports really send untagged frames.

## Updated Topology with Monitoring Port

PC7 connected to Gi3/1 is used as the SPAN destination port --> receives mirrored traffic.

## Complete SPAN Configuration

enable configure terminal monitor session 1 source interface Gi0/0 - 3 both monitor session 1 source interface Gi1/0 - 3 both monitor session 1 source interface Gi2/0 - 3 both monitor session 1 destination interface Gi3/1

## Verifying SPAN Session

show monitor session 1

## What You Should See in Wireshark

Access port traffic appears untagged

VLAN tag (TPID 0x8100) appears only on trunk links

Native VLAN traffic remains untagged even on a trunk

## Tag Behavior Summary Table

| Port Type | VLAN Tagging Behavior |
| --- | --- |
| Access Port | Always untagged |
| Trunk Port (Native VLAN) | Untagged |
| Trunk Port (Non-Native VLANs) | Tagged |

# Traffic Capture & VLAN Tag Analysis

## Access-Port Traffic --> Untagged

We will see normal Ethernet II frames - no “802.1Q Virtual LAN” line in Wireshark.

## Trunk Traffic --> Tagged (VLAN 10)

For Trunk Traffics there we should add a second switch --> make link trunk --> we will instantly see TPID 0x8100.

## 802.1Q Header Breakdown in Wireshark

Priority Code Point (PCP)

Drop Eligible Indicator (DEI)

VLAN Identifier (VID) → e.g., 10, 20, 30

## Native VLAN Traffic --> Untagged on Trunk

Even on a trunk, native VLAN frames appear without tags.

## Sample Packet Capture

# Key Observations & Learning Outcomes

Access ports always strip tags

Trunks preserve tags except for native VLAN

SPAN + Wireshark is the ultimate proof tool

Native VLAN mismatch = real production outages

# Best Practices & Security Recommendations

Never use VLAN 1 for user traffic

Change native VLAN on all trunks (switchport trunk native vlan 999)

Disable DTP: switchport nonegotiate

Explicitly set switchport mode access or trunk

Use dedicated management VLAN (not 1)

# Troubleshooting VLAN Issues

## Top 5 VLAN Problems on the Exam

Native VLAN mismatch

Trunk not allowing required VLANs

Access port in wrong VLAN

DTP negotiation failure

Using VLAN 1 for user traffic

## Native VLAN Mismatch Symptoms

%CDP-4-NATIVE_VLAN_MISMATCH logs

STP loops possible

## Must-Know Commands + Expected Output

show vlan brief show interfaces trunk show interfaces switchport show monitor session all

# Conclusion

This document provides a complete understanding of VLAN configuration, 802.1Q tag behavior, native VLAN operation, and SPAN-based packet analysis. The combination of theory and hands-on practice in GNS3 allows accurate visualization of VLAN operations and prepares learners for both CCNA-level exams and real-world troubleshooting